MODULE 4 ASSIGNMENT

1. What is the significance of MAC layer and in which position it is placed in the OSI model?

Position in OSI Model:

The MAC (Medium Access Control) layer is part of the:

- OSI Layer 2 – Data Link Layer

- Specifically, it is a sublayer of the Data Link Layer

Significance of MAC Layer:

The MAC layer plays a crucial role in Wi-Fi communication by controlling how devices access and share the wireless medium.

Key Functions of MAC Layer:

- Medium Access Control

  - Uses CSMA/CA to allow multiple devices to share the wireless channel efficiently

- Frame Formatting

  - Defines structure of frames (Data, Control, Management)

- Addressing

  - Uses MAC addresses to identify sender and receiver

- Error Detection

  - Uses Frame Check Sequence (FCS) to detect transmission errors

- Retransmission

  - Ensures reliable delivery using acknowledgments (ACKs)

- Fragmentation & Reassembly

  - Breaks large frames into smaller ones for efficient transmission

- QoS (Quality of Service)

  - Prioritizes traffic like voice and video

- Security Support

  - Works with encryption protocols (WPA2/WPA3)

Importance

- Ensures efficient and fair usage of wireless medium

- Provides reliable communication over an unreliable wireless channel

- Supports mobility and dynamic network behavior

2. Describe the frame format of the 802.11 MAC header and explain the purpose of each fields

The MAC frame consists of the following main parts:

| Field | Purpose |
| --- | --- |
| Frame Control (2 bytes) | Contains control information such as frame type (Management/Data/Control), subtype, flags (To DS, From DS, Retry, Power Management, etc.) |
| Duration (2 bytes) | Specifies time (in microseconds) the channel will be reserved (used for NAV – Network Allocation Vector) |
| Address 1 (6 bytes) | Receiver address (destination MAC) |
| Address 2 (6 bytes) | Transmitter address (source MAC) |
| Address 3 (6 bytes) | Used for routing (BSSID or AP address depending on frame type) |
| Sequence Control (2 bytes) | Contains sequence number and fragment number (used for ordering and reassembly) |
| Address 4 (6 bytes) | Used in special cases (e.g., WDS – Wireless Distribution System) |
| QoS Control (2 bytes) | Used in QoS-enabled frames to prioritize traffic |
| Frame Body (0–2312 bytes) | Contains actual data or management information |
| FCS (4 bytes) | Frame Check Sequence used for error detection |

Not all fields are always present (e.g., Address 4 and QoS Control are optional)

Frame structure varies slightly for:

- Management frames

- Control frames

- Data frames

3. Please list all the MAC layer functionalities in all Management, Control and Data plane

The MAC layer operations in WLAN are divided into three planes:

- Management Plane

- Control Plane

- Data Plane

| Plane | Functionality | Description |
| --- | --- | --- |
| Management Plane | Scanning | Discover available networks (Active/Passive) |
|  | Beaconing | AP broadcasts network information (SSID, capabilities) |
|  | Authentication | Verifies client identity before connection |
|  | Association / Reassociation | Establishes connection between client and AP |
|  | Disassociation / Deauthentication | Terminates connection |
|  | Roaming Support | Allows seamless movement between APs |
|  | Synchronization | Maintains timing using beacon frames |
|  | Power Management | Supports sleep/wake mechanisms |
| Control Plane | RTS/CTS | Avoids collisions using request-to-send/clear-to-send |
|  | ACK | Confirms successful frame reception |
|  | Block ACK | Acknowledges multiple frames together |
|  | NAV (Virtual Carrier Sense) | Reserves medium using duration field |
|  | Contention Handling | Uses CSMA/CA to manage access |
| Data Plane | Data Transmission | Transfers actual user data |
|  | Frame Aggregation | Combines frames (A-MSDU, A-MPDU) |
|  | Fragmentation | Splits large frames into smaller units |
|  | Error Detection | Uses FCS for integrity checking |
|  | Retransmission | Resends lost or corrupted frames |
|  | QoS Support | Prioritizes traffic (voice, video, data) |

4. Explain the scanning process and its types in detail

Scanning is the process by which a wireless client (STA) discovers available Wi-Fi networks (APs) before connecting.

Steps in Scanning Process:

- Initiation

  - Client decides to find available networks

- Channel Scanning

  - Client scans different channels in the frequency band

- AP Discovery

  - Detects nearby Access Points (APs)

- Information Collection

  - Collects details like:

    - SSID

    - Signal strength (RSSI)

    - Supported data rates

    - Security type

- Network Selection

  - Client selects the best AP based on signal and policies

Types of Scanning:

1. Passive Scanning

How it Works

- APs periodically send Beacon Frames

- Client listens on each channel and waits for beacons

Characteristics

- No transmission from client

- Slower process

- Less power consumption

Advantages

- Energy efficient

- Less network overhead

Disadvantages

- Takes more time

2. Active Scanning

How it Works

- Client sends Probe Request frames

- APs respond with Probe Response

Characteristics

- Faster discovery

- Client actively participates

Advantages

- Quick network detection

- Faster roaming

Disadvantages

- More power consumption

- Adds network traffic

5. Brief about the client association process

The client association process is the sequence of steps through which a wireless device (STA) connects to an Access Point (AP) and becomes part of the network.

Steps in Client Association:

| Step | Process | Description |
| --- | --- | --- |
| 1. Scanning | Network Discovery | Client scans for available APs (Active/Passive scanning) |
| 2. Authentication | Identity Verification | Client and AP perform authentication (Open/Shared/WPA/WPA2/WPA3) |
| 3. Association Request | Connection Request | Client sends association request to AP |
| 4. Association Response | Connection Approval | AP responds with success/failure and assigns Association ID (AID) |
| 5. Key Exchange (if secured) | Security Setup | WPA/WPA2 performs 4-way handshake for encryption keys |
| 6. Data Communication | Normal Operation | Client can now send/receive data |

- Association establishes a logical connection between client and AP

- Each client gets a unique Association ID (AID)

- Required before any data transmission

- Security (if enabled) is completed before full access

6. Explain each steps involved in EAPOL 4-way handshake and the purpose of each keys derived from the process

The 4-way handshake is used in WPA2/WPA3 security to establish secure communication between the client (STA) and the Access Point (AP).

Purpose

- Generate encryption keys

- Ensure both client and AP share the same secret

- Enable secure data transmission

Steps in 4-Way Handshake:

| Step | Message | Description |
| --- | --- | --- |
| 1 | AP → Client | AP sends ANonce (random number) |
| 2 | Client → AP | Client generates SNonce, derives keys, sends SNonce + MIC |
| 3 | AP → Client | AP verifies, sends GTK (Group Key) encrypted + MIC |
| 4 | Client → AP | Client confirms installation of keys |

Keys Derived in the Process:

| Key | Full Form | Purpose |
| --- | --- | --- |
| PMK | Pairwise Master Key | Pre-shared key (from password or authentication server) |
| PTK | Pairwise Transient Key | Derived during handshake; used for unicast encryption |
| GTK | Group Temporal Key | Used to encrypt broadcast/multicast traffic |
| ANonce | Authenticator Nonce | Random number generated by AP |
| SNonce | Supplicant Nonce | Random number generated by client |

How Keys Are Used

- PMK → Base key (already known to both)

- PTK → Derived using PMK + ANonce + SNonce + MAC addresses

- GTK → Sent by AP for group communication

Security Features

- Prevents replay attacks

- Ensures both sides prove knowledge of PMK

- Keys are dynamic and session-based

7. Describe the power saving scheme in MAC layer and explore on the types of Power saving mechanisms

Power saving mechanisms in WLAN are designed to reduce battery consumption of wireless devices (like mobiles, laptops) by allowing them to sleep when not actively transmitting/receiving data.

Basic Working:

- Client informs AP that it is entering power save mode

- AP buffers (stores) frames for that client

- Client periodically wakes up to check for buffered data

- If data is available, client retrieves it

Power Saving Mechanisms:

| Mechanism | Description | Use Case |
| --- | --- | --- |
| Legacy Power Save (PS-Poll) | Client wakes up and sends PS-Poll frame to request buffered data from AP | Basic power saving (older devices) |
| U-APSD (Unscheduled Automatic Power Save Delivery) | AP automatically sends buffered data when client triggers communication | Voice/video applications (low latency) |
| WMM Power Save | Enhanced version of U-APSD for QoS traffic | Multimedia traffic |
| TWT (Target Wake Time) | AP schedules specific wake-up times for clients | IoT devices, Wi-Fi 6 networks |

Key Components:

- TIM (Traffic Indication Map)

  - Included in beacon frames

  - Indicates which clients have buffered data

- DTIM (Delivery TIM)

  - Used for broadcast/multicast data delivery

Advantages:

- Reduces power consumption

- Extends battery life

- Efficient for mobile and IoT devices

8. Describe the Medium Access Control methodologies

Medium Access Control (MAC) methodologies define how multiple devices share the wireless medium efficiently without causing excessive collisions.

Main MAC Methodologies:

1. CSMA/CA (Core Mechanism)

Carrier Sense Multiple Access with Collision Avoidance

Primary method used in Wi-Fi to avoid collisions before transmission

Working Steps

- Device checks if channel is idle

- Waits for DIFS (Distributed Inter Frame Space)

- Starts random backoff timer

- If timer reaches zero → transmit

- If busy → wait again

Reduces collisions (since wireless cannot detect collisions like wired Ethernet)

2. DCF (Distributed Coordination Function)

Distributed Coordination Function

Basic access method based on CSMA/CA

- Default MAC mechanism

- Uses CSMA/CA

- All devices have equal priority

- Contention-based access

3. PCF (Point Coordination Function)

Point Coordination Function

Centralized polling-based access (rarely used)

- AP controls medium access

- Uses polling mechanism

- No contention

Rarely implemented in real networks

4. EDCA (QoS Support)

Enhanced Distributed Channel Access

QoS-based access method (priority handling)

- Extension of DCF

- Provides traffic prioritization:

| Voice | - Highest |
| --- | --- |
| Video | - High |
| Best Effort | - Medium |
| Background | - Lowest |

Uses different backoff times for priority

5. HCF (Hybrid Coordination Function)

Hybrid Coordination Function

Combines DCF and PCF with QoS support

- Combines DCF + PCF

- Supports QoS and controlled access

- Used in modern Wi-Fi (802.11e)

9. Brief about the Block ACK mechanism and its advantages

Block Acknowledgment (Block ACK) is a MAC layer mechanism used to acknowledge multiple data frames at once instead of individually.

How Block ACK Works:

- Sender transmits multiple frames in a burst

- Receiver collects the frames

- Receiver sends a single Block ACK frame

- Block ACK contains a bitmap indicating:

  - Which frames were received successfully

  - Which frames are missing

Types of Block ACK:

- Immediate Block ACK -  ACK is sent immediately after receiving frames

- Delayed Block ACK - ACK is sent after some delay

Advantages

- Reduces Overhead

  - Fewer ACK frames → less control traffic

- Improves Throughput

  - More data transmitted in less time

- Efficient Retransmission

  - Only lost frames are retransmitted (based on bitmap)

- Better Performance in High Traffic

  - Suitable for high-speed networks (Wi-Fi 4 and above)

10. Explain about A-MSDU, A-MPDU and A-MSDU in A-MPDU

Frame aggregation is used to combine multiple frames into a single transmission, reducing overhead and improving efficiency.

Types of Aggregation:

1. A-MSDU

- Aggregated MAC Service Data Unit

- Multiple MSDUs are combined into one MPDU

- Single MAC header for all data

Characteristics

- Low overhead

- Less reliable (if error occurs → entire frame retransmitted)

2. A-MPDU

- Aggregated MAC Protocol Data Unit

- Multiple MPDUs are aggregated together

- Each MPDU has its own MAC header and FCS

Characteristics

- More reliable

- Supports Block ACK

- Only failed MPDUs are retransmitted

3. A-MSDU in A-MPDU

- Hybrid Aggregation

- Combines both techniques:

  - MSDUs aggregated → form MPDUs

  - Multiple MPDUs aggregated → form A-MPDU

Characteristics

- Maximum efficiency

- High throughput

- Balanced reliability and overhead