MODULE 5 ASSIGNMENT

1. What are the key features of Wi-Fi 6, 6E and 7 and how do they differ from previous

standards like Wi-Fi 5 (802.11ac)?

Wi-Fi standard overview:

| Standard | Generation | Frequency Bands |
| --- | --- | --- |
| 802.11ac | Wi-Fi 5 | 5 GHz |
| 802.11ax | Wi-Fi 6 | 2.4 & 5 GHz |
| 802.11ax (6E) | Wi-Fi 6E | 2.4, 5 & 6 GHz |
| 802.11be | Wi-Fi 7 | 2.4, 5 & 6 GHz |

Key features comparisons:

| Feature | Wi-Fi 5 (802.11ac) | Wi-Fi 6 (802.11ax) | Wi-Fi 6E | Wi-Fi 7 (802.11be) |
| --- | --- | --- | --- | --- |
| Frequency Bands | 5 GHz only | 2.4 & 5 GHz | Adds 6 GHz | 2.4, 5, 6 GHz |
| Max Speed | ~6.9 Gbps | ~9.6 Gbps | ~9.6 Gbps | ~46 Gbps |
| Modulation | 256-QAM | 1024-QAM | 1024-QAM | 4096-QAM (4K-QAM) |
| Channel Width | Up to 160 MHz | Up to 160 MHz | Up to 160 MHz | Up to 320 MHz |
| OFDMA | Not supported | Supported | Supported | Enhanced |
| MU-MIMO | Downlink only | Uplink + Downlink | Same as Wi-Fi 6 | Enhanced MU-MIMO |
| Latency | Moderate | Lower | Lower | Very low |
| Efficiency | Medium | High (dense networks) | Higher (less congestion) | Extremely high |
| Special Features | Beamforming | TWT, BSS Coloring | Clean spectrum | MLO, Multi-RU, Preamble Puncturing |

Key Improvements:

Wi-Fi 6 (over Wi-Fi 5)

- Introduced OFDMA → better multi-user efficiency

- Supports 1024-QAM → higher data rates

- Added TWT (Target Wake Time) → power saving

- Improved performance in dense environments

Wi-Fi 6E (over Wi-Fi 6)

- Adds 6 GHz band

- More channels → less interference

- Better for high-speed, low-latency applications

Wi-Fi 7 (over Wi-Fi 6/6E)

- Introduces Multi-Link Operation (MLO)

- Supports 320 MHz channels

- Uses 4096-QAM for ultra-high speed

- Extremely low latency and high throughput

2. Explain the role of OFDMA in Wi-Fi 6 and how it improves network efficiency.

OFDMA is a key feature introduced in Wi-Fi 6 (802.11ax) that allows multiple users to share the same channel simultaneously.

How OFDMA Works

- A channel is divided into smaller units called Resource Units (RUs)

- Each RU is assigned to different users

- Multiple devices can transmit/receive data at the same time

Unlike OFDM (single user), OFDMA supports multi-user transmission

Role of OFDMA in Wi-Fi 6:

- Enables parallel communication with multiple devices

- Efficiently utilizes available bandwidth

- Reduces waiting time for devices

How OFDMA Improves Efficiency:

- Better Spectrum Utilization

  - Channel is divided and used efficiently

- Reduced Latency

  - Devices don’t have to wait for full channel access

- Improved Performance in Dense Networks

  - Works well in environments with many devices (offices, campuses)

- Lower Contention

  - Reduces collisions compared to CSMA/CA

- Supports Small Data Packets

  - Ideal for IoT devices sending small amounts of data

Example:

- Without OFDMA:

  - One device uses entire channel → others wait

- With OFDMA:

  - Channel divided → multiple devices transmit simultaneously

Comparison:

| Feature | OFDM | OFDMA |
| --- | --- | --- |
| Users per transmission | One | Multiple |
| Efficiency | Lower in dense networks | High |
| Latency | Higher | Lower |
| Channel Usage | Full channel | Divided (RUs) |

3. Discuss the benefits of Target Wake Time (TWT) in Wi-Fi 6 for loT devices.

TWT is a power-saving feature introduced in Wi-Fi 6 (802.11ax) that allows devices to schedule specific times to wake up and communicate with the Access Point (AP).

How TWT Works:

- AP and client agree on a wake-up schedule

- Device stays in sleep mode most of the time

- Wakes up only at scheduled intervals to send/receive data

- Goes back to sleep after communication

Benefits of TWT for IoT Devices:

- Reduced Power Consumption

  - Devices sleep longer → saves battery

  - Ideal for battery-operated IoT devices

- Extended Battery Life

  - IoT devices can last for months/years

- Reduced Network Congestion

  - Devices transmit only at scheduled times

  - Avoids unnecessary channel usage

- Improved Efficiency

  - Organized communication → less contention

- Lower Latency for Scheduled Traffic

  - Data transmission happens at predictable intervals

Use Cases:

- Smart home devices (sensors, smart bulbs)

- Wearables

- Industrial IoT systems

- Smart meters

Comparison with Traditional power save:

| Feature | Traditional Power Save | TWT |
| --- | --- | --- |
| Wake-up pattern | Random/periodic | Scheduled |
| Efficiency | Moderate | High |
| Battery life | Limited | Extended |
| Network usage | Less optimized | Highly optimized |

4. Explain the significance of the 6 GHz frequency band in Wi-Fi 6E.

6 GHz Band in Wi-Fi 6E:

Wi-Fi 6E extends Wi-Fi 6 by introducing a new frequency band:
6 GHz (5.925 GHz – 7.125 GHz)

Significance of 6 GHz Band:

- More Spectrum Availability

  - Provides a large amount of new bandwidth

  - Many additional channels available

- Less Interference

  - No legacy devices (unlike 2.4 GHz & 5 GHz)

  - Cleaner spectrum → better performance

- Supports Wider Channels

  - Enables 80 MHz, 160 MHz channels without overlap

  - Higher data rates

- Higher Throughput

  - More bandwidth → faster speeds

- Low Latency

  - Reduced congestion → faster communication

Channel Advantages:

- More non-overlapping channels

- Better for high-density environments

- Ideal for applications requiring high bandwidth

Use Cases:

- AR/VR applications

- 4K/8K video streaming

- Gaming

- Enterprise high-density deployments

Limitations:

- Shorter range

- Poor wall penetration

5. Compare and contrast Wi-Fi 6 and Wi-Fi 6E in terms of range, bandwidth, and

interference.

Wi-Fi 6 vs Wi-Fi 6E:

Wi-Fi 6 and Wi-Fi 6E use the same technology (802.11ax), but Wi-Fi 6E extends it by adding the 6 GHz band.

Range

- Wi-Fi 6:

  - Operates on 2.4 GHz and 5 GHz

  - Offers better range, especially on 2.4 GHz

- Wi-Fi 6E:

  - Operates on 6 GHz (in addition to others)

  - Has shorter range and weaker wall penetration

Bandwidth:

- Wi-Fi 6:

  - Limited by crowded 2.4 GHz and 5 GHz spectrum

  - Fewer wide channels available

- Wi-Fi 6E:

  - Access to large new spectrum in 6 GHz

  - Supports more 80 MHz and 160 MHz channels

  - Provides higher throughput

Interference:

- Wi-Fi 6:

  - Faces interference from legacy devices

  - Congestion in busy environments

- Wi-Fi 6E:

  - Much lower interference

  - No legacy devices in 6 GHz → cleaner communication

Key Insight:

- Wi-Fi 6 = better coverage + compatibility

- Wi-Fi 6E = better performance + less congestion

Comparison:

| Feature | Wi-Fi 6 (802.11ax) | Wi-Fi 6E (802.11ax extended) |
| --- | --- | --- |
| Frequency Bands | 2.4 GHz & 5 GHz | 2.4 GHz, 5 GHz & 6 GHz |
| Range | Longer range (especially 2.4 GHz) | Shorter range (6 GHz has poor penetration) |
| Wall Penetration | Better | Weaker (higher frequency) |
| Bandwidth Availability | Limited (crowded spectrum) | Very high (new 6 GHz spectrum) |
| Channel Availability | Fewer wide channels | Many 80/160 MHz channels |
| Interference | Moderate to high (legacy devices present) | Very low (clean spectrum, no legacy devices) |
| Network Congestion | Higher in dense environments | Much lower due to extra spectrum |
| Performance | Good | Better (especially in dense networks) |

6. What are the major innovations introduced in Wi-Fi 7 (802.11be)?

Wi-Fi 7 (802.11be) introduces several advanced technologies to achieve ultra-high speed, extremely low latency, and better efficiency.

Major Innovations in Wi-Fi 7:

- Multi-Link Operation (MLO)

  - Allows devices to use multiple frequency bands (2.4, 5, 6 GHz) simultaneously

  - Improves speed, reliability, and reduces latency

- 320 MHz Channel Width

  - Doubles channel width compared to Wi-Fi 6 (160 MHz)

  - Enables significantly higher data rates

- 4096-QAM (4K QAM)

  - Higher modulation than Wi-Fi 6 (1024-QAM)

  - Transmits more bits per symbol → higher throughput

- Multi-RU (Resource Unit) Allocation

  - Allows multiple resource units to be assigned to a single user

  - Improves efficiency and flexibility

- Preamble Puncturing

  - Enables partial use of a channel even if part of it is interfered

  - Avoids wasting entire bandwidth

- Enhanced MU-MIMO

  - Supports more spatial streams (up to 16x16)

  - Improves multi-user performance

- Extremely Low Latency

  - Designed for real-time applications like AR/VR and gaming

Key Benefits:

- Much higher throughput (up to ~46 Gbps)

- Better performance in dense environments

- Lower latency and improved reliability

7. Explain the concept of Multi-Link Operation (MLO) and its impact on throughput and latency.

MLO is a key feature introduced in Wi-Fi 7 (802.11be) that allows a device to use multiple frequency bands or channels simultaneously instead of relying on a single link.

How MLO Works:

- A device can connect using multiple links across:

  - 2.4 GHz

  - 5 GHz

  - 6 GHz

- Data can be:

  - Split across multiple links (parallel transmission)

  - Or switched dynamically based on channel conditions

Types of MLO Operation:

- Simultaneous Transmission

  - Data sent over multiple links at the same time

- Load Balancing

  - Traffic distributed across links

- Link Switching

  - Switches to best link if one degrades

Impact on Throughput:

- Significantly Increased Speed

  - Multiple links → parallel data transfer

- Better Bandwidth Utilization

  - Uses all available spectrum efficiently

Result: Higher overall throughput

Impact on Latency

- Reduced Latency

  - Data can take the fastest available path

- Less Delay

  - No need to wait for a single channel

- Improved Reliability

  - If one link fails, others continue

Result: Ultra-low latency communication

Key Advantages

- Faster data transmission

- More stable connections

- Reduced congestion impact

- Better performance in dynamic environments

8. What is the purpose of 802.11k and v, and how does it aid in roaming?

802.11k (Radio Resource Management - RRM)

Purpose:

- Helps clients identify the best available Access Points (APs)

How it Works:

- AP provides a neighbor report (list of nearby APs)

- Client gets information like:

  - Signal strength

  - Channel details

  - AP load

Benefit for Roaming:

- Client doesn’t need to scan all channels

- Faster and more efficient AP selection

802.11v (Wireless Network Management - WNM)

Purpose:

- Allows the network (AP/controller) to guide client roaming decisions

How it Works:

- AP suggests better APs using:

  - BSS Transition Management (BTM)

- Helps move clients to:

  - Less congested APs

  - Better signal APs

Benefit for Roaming:

- Smarter and controlled roaming

- Improves load balancing and user experience

How They Aid in Roaming:

- 802.11k → Provides information (neighbor APs)

- 802.11v → Guides decision (which AP to move to)

Together:

- Reduce scanning delay

- Improve roaming speed

- Enhance network performance

9. Explain the concept of Fast BSS Transition (802.11r) and its benefit in mobile

environments.

802.11r (Fast BSS Transition - FT)

802.11r is a standard designed to speed up the roaming process between Access Points (APs) by reducing authentication delay.

Problem Without 802.11r

- When a client moves from one AP to another:

  - It must re-authenticate and re-establish keys

  - This causes delay (latency)

  - Leads to packet loss or call drops (especially in VoIP/video)

How 802.11r Works

- Allows pre-authentication with the target AP

- Keys are derived and shared in advance

- When roaming happens:

  - Client quickly switches AP

  - No full authentication needed again

Two methods:

- Over-the-Air (direct communication with new AP)

- Over-the-DS (through distribution system)

Benefits in Mobile Environments

- Faster Roaming

  - Reduces handoff time significantly

- Low Latency

  - Suitable for real-time applications

- Seamless Connectivity

  - No noticeable interruption

- Better User Experience

  - Smooth voice/video calls

Use Cases

- VoIP (Wi-Fi calling)

- Video conferencing

- Enterprise/campus networks

- Hospitals, warehouses

10. How do 802.11k/v/r work together to provide seamless roaming in enterprise networks?

Combined Role of 802.11k, 802.11v, and 802.11r

These three standards work together to provide fast, intelligent, and seamless roaming in WLANs.

Individual Roles

- 802.11k → Information

  - Provides list of nearby APs (neighbor report)

  - Helps client quickly identify better APs

- 802.11v → Decision Guidance

  - AP suggests the best AP for the client

  - Helps with load balancing and optimal selection

- 802.11r → Fast Transition

  - Speeds up authentication during roaming

  - Reduces handoff delay

How They Work Together

- Discovery (802.11k)

  - Client gets information about nearby APs

- Decision (802.11v)

  - Network guides client to the best AP

- Transition (802.11r)

  - Client quickly connects with minimal delay

End Result

- Faster roaming

- Reduced latency

- Minimal packet loss

- Seamless user experience

Example Scenario

- A user moves in a campus while on a video call:

  - 802.11k → Finds nearby APs

  - 802.11v → Suggests best AP

  - 802.11r → Connects instantly

No call drop or interruption