MODULE 6 ASSIGNMENT

1. What are the pillars of Wi-Fi security?

Pillars of Wi-Fi Security

Wi-Fi security is built on three fundamental pillars:

1. Authentication

- Verifies the identity of users/devices before allowing access

- Ensures only authorized users can connect

Examples:

- Password-based (WPA2-PSK)

- Enterprise authentication (802.1X, RADIUS)

2. Encryption (Confidentiality)

- Protects data transmitted over the air

- Converts plain data into encrypted format

Prevents:

- Eavesdropping

- Data interception

Examples:

- TKIP, AES, GCMP

3. Integrity

- Ensures data is not altered during transmission

- Detects tampering or modification

Uses:

- Message Integrity Check (MIC)

Additional Supporting Pillars (sometimes included)

- Availability

  - Network should be accessible to authorized users

- Access Control

  - Controls who can access network resources

2. Explain the difference between authentication and encryption in WiFi security.

Authentication vs Encryption:

| Feature | Authentication | Encryption |
| --- | --- | --- |
| Purpose | Verifies identity of user/device | Protects data during transmission |
| Focus | “Who are you?” | “What data is being sent?” |
| When it occurs | Before network access is granted | After authentication, during data transfer |
| Function | Allows or denies access | Secures communication |
| Example Methods | WPA2-PSK, WPA3, 802.1X | AES, TKIP, GCMP |
| Security Goal | Access control | Confidentiality |

Explanation

- Authentication

  - Ensures only authorized users connect

  - Happens during connection setup

- Encryption

  - Scrambles data so others cannot read it

  - Happens continuously during communication

Simple Analogy

- Authentication → Checking ID at a gate

- Encryption → Locking your conversation so others can’t hear

3. Explain the differences between WEP, WPA, WPA2, and WPA3.

| Feature | WEP | WPA | WPA2 | WPA3 |
| --- | --- | --- | --- | --- |
| Full Form | Wired Equivalent Privacy | Wi-Fi Protected Access | Wi-Fi Protected Access 2 | Wi-Fi Protected Access 3 |
| Encryption | RC4 | TKIP (RC4-based) | AES (CCMP) | AES (GCMP), stronger crypto |
| Security Level | Very Weak | Improved over WEP | Strong | Very Strong |
| Key Management | Static keys | Dynamic keys | Dynamic keys | Enhanced key exchange (SAE) |
| Authentication | Open / Shared | PSK / 802.1X | PSK / 802.1X | SAE / 802.1X |
| Vulnerability | Easily breakable | Some vulnerabilities | Secure (with proper config) | Highly secure, resistant to attacks |
| Protection Against Attacks | None | Basic protection | Strong protection | Protection against brute-force & offline attacks |
| Usage | Obsolete | Deprecated | Widely used | Latest standard |

Key Differences Explained

- WEP

  - Uses weak encryption (RC4)

  - Easily cracked → not used anymore

- WPA

  - Temporary improvement over WEP

  - Introduced TKIP for better security

- WPA2

  - Uses AES encryption (CCMP)

  - Became the standard for secure Wi-Fi

- WPA3

  - Uses SAE (Simultaneous Authentication of Equals)

  - Provides stronger protection against password attacks

  - Better security even with weak passwords

4. Why is WEP considered insecure compared to WPA2 or WPA3?

Reasons Why WEP is Insecure:

- Weak Encryption Algorithm (RC4)

  - Uses outdated RC4 cipher

  - Easily broken with modern tools

- Static Keys

  - Same key is used for long periods

  - No automatic key update

- Weak Initialization Vector (IV)

  - IV is only 24 bits

  - Repeats quickly → attackers can analyze patterns

- No Proper Key Management

  - No secure key exchange mechanism

  - Keys can be easily captured

- No Strong Integrity Protection

  - Uses weak CRC for error checking

  - Can be modified without detection

- Susceptible to Attacks

  - Packet sniffing

  - Replay attacks

  - Key cracking within minutes

Why WPA2 / WPA3 are Secure

- WPA2

  - Uses AES encryption (CCMP)

  - Strong key management

  - Secure 4-way handshake

- WPA3

  - Uses SAE authentication

  - Resistant to brute-force and offline attacks

  - Stronger encryption and forward secrecy

5. Why was WPA2 introduced?

Reason for Introducing WPA2:

WPA2 was introduced to overcome the security weaknesses of WEP and WPA and to provide a strong, standardized Wi-Fi security framework.

Problems with Previous Standards:

- WEP

  - Weak encryption (RC4)

  - Easily breakable

  - Poor key management

- WPA

  - Used TKIP, which was a temporary fix

  - Still relied on RC4 → not fully secure

Improvements Introduced in WPA2:

- Strong Encryption (AES-CCMP)

  - Replaced RC4/TKIP

  - Provides high-level security

- Robust Key Management

  - Secure 4-way handshake

  - Dynamic key generation

- Better Data Integrity

  - Strong integrity checks prevent tampering

- Support for Enterprise Security

  - Works with 802.1X and RADIUS authentication

- Standardization

  - Based on IEEE 802.11i standard

Benefits of WPA2:

- Much stronger protection against attacks

- Secure communication over wireless networks

- Widely adopted and reliable

6. What is the role of the Pairwise Master Key (PMK) in the 4-way handshake?

PMK is the main secret key shared between the client (STA) and the Access Point (AP) before the 4-way handshake begins.

How PMK is Obtained:

- WPA2-PSK (Personal mode):

  - Derived from the Wi-Fi password (passphrase)

- WPA2-Enterprise (802.1X):

  - Generated during authentication via RADIUS server

Role of PMK in 4-Way Handshake:

- Used to derive the Pairwise Transient Key (PTK)

- Combined with:

  - ANonce (from AP)

  - SNonce (from client)

  - MAC addresses

PMK never travels over the network (for security)

Key Functions:

- Key Generation

  - Acts as base for generating encryption keys (PTK)

- Authentication Verification

  - Ensures both AP and client have the same secret

- Security Foundation

  - Root of trust for the entire session

Why PMK is Important:

- If PMK is secure → communication is secure

- If PMK is compromised → entire connection is vulnerable

7. How does the 4-way handshake ensure mutual authentication between the client and the access point?

Mutual Authentication in 4-Way Handshake

The 4-way handshake ensures that both the client (STA) and Access Point (AP) verify each other by proving they possess the same Pairwise Master Key (PMK).

How It Achieves Mutual Authentication:

- Both AP and client already have the PMK (from PSK or 802.1X)

- During the handshake, they exchange nonces and MIC values

- Each side validates the other using derived keys

Step-by-Step Authentication Logic:

- AP sends ANonce

  - Initiates handshake

  - Client uses it to derive PTK

- Client sends SNonce + MIC

  - MIC is generated using PTK (derived from PMK)

  - AP verifies MIC → confirms client knows PMK

- AP sends GTK + MIC

  - AP sends group key securely

  - Client verifies MIC → confirms AP knows PMK

- Client sends final ACK

  - Confirms successful key installation

Key Mechanism:

- MIC (Message Integrity Code)

  - Used to verify authenticity

  - If MIC matches → sender is valid

Why This Ensures Mutual Authentication

- Client proves to AP → by generating valid MIC

- AP proves to client → by sending valid MIC

Both sides confirm they share the same PMK

8. What will happen if we put a wrong passphrase during a 4Way handshake?

Effect of Wrong Passphrase

If a wrong passphrase is entered, the Pairwise Master Key (PMK) derived by the client will be different from the AP’s PMK.

What Happens During Handshake:

- AP sends ANonce → Client proceeds normally

- Client generates PTK using wrong PMK

- Client sends SNonce + MIC

- AP verifies MIC → fails

Because PTK is incorrect, the MIC will not match

Result:

- Authentication Failure

  - AP rejects the client

- Handshake Fails

  - 4-way handshake does not complete

- Client Cannot Connect

  - No network access granted

- Retry Attempts

  - Client may retry connection multiple times

Key Reason

- PMK must match on both sides

- Wrong password → different PMK → invalid MIC

9. What problem does 802.1X solve in a network?

Problem in Traditional Wi-Fi Security:

In networks using pre-shared keys (PSK):

- All users share the same password

- Difficult to manage in large organizations

- If one user leaves → password must be changed for everyone

- No individual user accountability

Problem Solved by 802.1X

802.1X provides centralized authentication and access control.

Key Problems It Solves

- No Individual Authentication → Solved

  - Each user gets unique credentials

- Poor Security in PSK → Solved

  - Eliminates shared passwords

- Lack of Access Control → Solved

  - Only authorized users can connect

- No User Tracking → Solved

  - Enables logging and accountability

- Manual Management → Solved

  - Uses centralized RADIUS server

How 802.1X Works (Brief)

- Uses:

  - Client (Supplicant)

  - Access Point (Authenticator)

  - Authentication Server (RADIUS)

- Performs secure authentication before granting access

Benefits

- Strong enterprise-level security

- Scalable for large networks

- Easy user management

- Dynamic key generation

10. How does 802.1X enhance security over wireless networks?

802.1X in Wireless Security

802.1X enhances Wi-Fi security by providing centralized, per-user authentication and dynamic key management, making networks much more secure than PSK-based systems.

How It Enhances Security:

- Individual User Authentication

  - Each user logs in with unique credentials (username/password or certificate)

  - Eliminates shared passwords

- Centralized Authentication (RADIUS)

  - Authentication handled by a RADIUS server

  - Easy to manage users and policies

- Dynamic Key Generation

  - Generates unique session keys for each user

  - Keys change per session → higher security

- Stronger Encryption

  - Works with WPA2/WPA3 Enterprise

  - Ensures secure data transmission

- Access Control

  - Can enforce policies (who can access what)

  - Supports role-based access

- Improved Accountability

  - Tracks user activity

  - Useful for auditing and monitoring

Comparison with PSK:

- PSK → One shared password (less secure)

- 802.1X → Unique credentials + dynamic keys (more secure)